| Technical Specs & Quotation Form(Off-grid system) see fig 1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | HY-2KW-Off-Grid | HY-5KW-Off-Grid | HY-10KW-Off-Grid | HY-20KW-Off-Grid |  |  | HY-25KW-Off-Grid |  |  | HY-30KW-Off-Grid |  |
| Rated power(KW) | 2KW | 5KW | 10KW | 20KW |  |  | 25KW |  |  | 30KW |  |
| Rotor diameter(m) | 4M | 5.6 M | 7.8 M | 10 M |  |  | 10 M |  |  | 11M |  |
| Rated RPM(r/min) | 360R | 240R | 200R | 185R |  |  | 185R |  |  | 120R |  |
| Rated voltage(V) | DC48V/AC220V | DC240V/AC220V | DC300V/ AC220V | DC500V/AC380V |  |  | DC540V/ AC380V |  |  | DC540V/ AC380V |  |
| Rated wind speed(m/s) | 9 m/s | 11 m/s | 11 m/s | 12 m/s |  |  | 12 m/s |  |  | 12m/s |  |
| Starting wind speed(m/s) | 2.5 m/s | 3 m/s | 3 m/s | 3 m/s |  |  | 3 m/s |  |  | 3m/s |  |
| Working wind speed(m/s) | 3~25 m/s |  |  | 4~25 m/s |  |  |  |  |  |  |  |
| Generator type | Permanent Magnet 3 Phase AC |  |  |  |  |  |  |  |  |  |  |
| Blade material | Reinforced Fiberglass |  |  |  |  |  |  |  |  |  |  |
| Speed regulation | Variable pitch |  |  | Yawing & Mechanical brake |  |  |  | Variable pitch |  |  |  |
| Wind direction | Down wind without Tail vane |  |  | Up wind with Tail vane |  |  |  | Down wind without Tail vane |  |  |  |
| Tower height(m) | 6M | 8 M | 10 M | 15 M |  | 15 M |  | 15M |  |  |  |
| Wind turbine weight(kg) | 130 kg | 340 kg | 500 kg | 1360 kg |  | 1370 kg |  | 1770kg |  |  |  |
| System price FOB（$） | Quotation of wind turbine |  |  |  |  |  |  |  |  |  |  |
| Wind turbine body FOB $ |  |  |  |  |  |  |  |  |  |  |  |
| Tower type | Freestanding tower | Freestanding tower | Freestanding tower | Freestanding tower | Freestanding tower |  |  |  | Freestanding tower |  |  |
| Tower&Accessory FOB $ |  |  |  |  |  |  |  |  |  |  |  |
| Off-Controller FOB $ |  |  |  |  |  |  |  |  |  |  |  |
| Off-Inverter FOB $ |  |  |  |  |  |  |  |  |  |  |  |
| Amount FOB $ |  |  |  |  |  |  |  |  |  |  |  |
| Technical Specs & Quotation Form (On-grid system) see fig 2 |  |  |  |  |  |  |  |  |  |  |  |
| Model | HY-2KW-On-Grid | HY-5KW-On-Grid | HY-10KW-On-Grid | HY-20KW-On-Grid |  |  | HY-25KW-On-Grid |  |  | HY-30KW-On-Grid |  |
| Rated power(KW) | 2KW | 5KW | 10KW | 20KW |  |  | 25KW |  |  | 30KW |  |
| Rotor diameter(m) | 4M | 5.6 M | 7.8 M | 10 M |  |  | 10 M |  |  | 11M |  |
| Rated RPM(r/min) | 360R | 240R | 200R | 185R |  |  | 185R |  |  | 120R |  |
| Rated voltage(V) | DC48V/AC220V | DC240V/AC220V | DC300V/ AC220V | DC500V/AC380V |  |  | DC540V/ AC380V |  |  | DC540V/ AC380V |  |
| Rated wind speed(m/s) | 9 m/s | 11 m/s | 11 m/s | 12 m/s |  |  | 12 m/s |  |  | 12m/s |  |
| Starting wind speed(m/s) | 2.5 m/s | 3 m/s | 3 m/s | 3 m/s |  |  | 3 m/s |  |  | 3m/s |  |
| Working wind speed(m/s) | 3~25 m/s |  |  | 4~25 m/s |  |  |  |  |  |  |  |
| Generator type | Permanent Magnet 3 Phase AC |  |  |  |  |  |  |  |  |  |  |
| Blade material | Reinforced Fiberglass |  |  |  |  |  |  |  |  |  |  |
| Speed regulation | Variable pitch |  |  | Yawing & Mechanical brake |  |  |  | Variable pitch |  |  |  |
| Wind direction | Down wind without Tail vane |  |  | Up wind with Tail vane |  |  |  | Down wind without Tail vane |  |  |  |
| Tower height(m) | 6M | 8 M | 10 M | 15 M |  | 15 M |  | 15M |  |  |  |
| Wind turbine weight(kg) | 130 kg | 340 kg | 500 kg | 1360 kg |  | 1370 kg |  | 1770kg |  |  |  |
| System price FOB（$） | Quotation of wind turbine |  |  |  |  |  |  |  |  |  |  |
| Wind turbine body FOB$ |  |  |  |  |  |  |  |  |  |  |  |
| Tower type | Freestanding tower | Freestanding tower | Freestanding tower | Freestanding tower | Freestanding tower |  |  |  | Freestanding tower |  |  |
| Tower&Accessory FOB$ |  |  |  |  |  |  |  |  |  |  |  |
| On-Controller FOB $ |  |  |  |  |  |  |  |  |  |  |  |
| On-Inverter FOB$ |  |  |  |  |  |  |  |  |  |  |  |
| Amount FOB $ |  |  |  |  |  |  |  |  |  |  |  |